GLOBAL PRECISION WORKS

Server Rack Assembly Plant

Best Practices Guide

| TARGET CAPACITY 50-600 racks/mo | FACILITY SIZE 15,000-50,000 sq ft | BURN-IN CAPACITY 10-50+ racks |
| --- | --- | --- |

Internal Reference Document  |  Version 1.0  |  March 2026

Monterrey, Nuevo León, México

# TABLE OF CONTENTS

# Global Precision Works (GPW) — Internal Reference Document

Document Version: 1.0

Date: March 2026

Purpose: Comprehensive operational reference for designing, building, and operating a high-volume server rack assembly facility in Monterrey, Mexico

Audience: Operations, Facility Planning, Engineering, Quality Teams

# TABLE OF CONTENTS

- Executive Summary

- Industry Benchmarks (ODM/OEM Analysis)

- Plant Zones — Complete Specifications

- Assembly Workflow — Step by Step

- Material Flow & Plant Layout

- Burn-in Room Deep Dive

- ESD Program Requirements

- Quality Standards & Certifications

- Environmental & Safety Standards

- Lean Manufacturing Integration

- Capacity Planning

- Monterrey-Specific Considerations

- Appendices

# 1. EXECUTIVE SUMMARY

## Purpose & Scope

This document provides exhaustive specifications for designing an electromechanical assembly facility focused on server rack integration and testing. The facility must support high-volume, high-precision assembly (5–600 racks/day depending on phase) with rigorous quality control and burn-in testing capabilities. All recommendations are based on industry best practices, ANSI/ESD standards, ASHRAE specifications, and manufacturing engineering principles validated through research of leading ODMs (Supermicro, Wistron, QCT, Foxconn, Celestica, Inventec, Flex, ZT Systems).

## Key Metrics for Target Production Levels

Phase 1 (Year 1) — 50–100 racks/month:

- Total facility footprint: 15,000–25,000 sq ft (1,400–2,300 m²)

- Core production area: 6,000–10,000 sq ft

- Personnel requirements: 25–45 technicians

- Power requirements: 300–500 kW

- HVAC cooling capacity: 50–150 W/sq ft in assembly and test areas

- Simultaneous burn-in capacity: 10–20 racks

Phase 2 (Years 2–3) — 200–300 racks/month:

- Facility footprint: 25,000–35,000 sq ft

- Personnel: 250–350

- Power: 2–3 MW

- Simultaneous burn-in: 30–40 racks

Phase 3 (Years 4–5) — 400–600 racks/month:

- Facility footprint: 35,000–50,000 sq ft

- Personnel: 500–700

- Power: 3–5 MW

- Simultaneous burn-in: 50+ racks

## Key Success Factors

- Rigorous ESD control (ANSI/ESD S20.20 compliance mandatory)

- Burn-in testing capacity as primary differentiator

- Lean manufacturing with takt time discipline

- Multi-shift operational capability

- Quality certifications (ISO 9001, IPC-A-610, industry-specific)

- Advanced MES integration for traceability

# 2. INDUSTRY BENCHMARKS (ODM/OEM ANALYSIS)

## 2.1 Supermicro (San Jose & Taiwan)

Facility Overview:

- San Jose campus: 182,000 sq ft (16,900 m²), LEED Gold certified

- Building 21 (Rack Integration): 100,000+ sq ft dedicated facility

- Taiwan expansion: 800,000+ sq ft (facility opened 2021)

- Global capacity: ~81,000 servers/month

Key Capabilities:

- Rack assembly capacity: 600 racks/month (San Jose Building 21)

- Simultaneous burn-in: 60 racks (industry-leading)

- Power supply: 3 MW from Bloom Energy fuel cells (clean energy)

- Automation: AGV robots with arm-like paddles for rack transport, reflective wall sensors for navigation, conveyor systems

- Assembly time: 20–30 minutes per server

- Testing time: 10–20 minutes per server

Key Differentiation:

- Vertical integration: In-house robotics design and deployment

- Co-location model: Design, engineering, and manufacturing reduce design-to-production cycle

- Burn-in at scale: 60-rack simultaneous capacity

- Clean energy: Fuel-cell powered facility supports hyperscaler sustainability credentials

Lessons for GPW:

- Invest in overhead power distribution (torque-controlled screwdrivers hanging from rails)

- AGV systems pay off at >200 racks/month; justify in Phase 2–3

- Simultaneous burn-in capacity is market differentiator; size conservatively (30–40 racks for Phase 2)

## 2.2 Wistron

Facility Overview:

- Employees: 80,000+ globally

- Manufacturing bases: 12 globally

- Server production: 240,000 Blackwell-based systems/quarter (as of 2025)

- Market focus: AI servers, Nvidia platforms

U.S. Facilities:

- Dallas: Server assembly (ready for Nvidia Blackwell/Rubin by 2026)

- Houston: Jointly operated with Foxconn for module integration and system-level testing

- Fort Worth Alliance: $761M investment, 800+ jobs, two AI supercomputer manufacturing plants (operational 2026)

Key Differentiation:

- Rapid U.S. expansion: Three-continent presence with dedicated Nvidia capacity

- Revenue growth: 200%+ increase in accelerated systems revenue (2024–2025)

- Digital optimization: Omniverse simulation achieved 51% efficiency improvement in bottleneck production lines

- Real-time monitoring: Virtual dashboard showing production status across all workstations

Lessons for GPW:

- Digital twins and simulation reduce ramp-up time significantly

- Real-time production visibility drives continuous improvement

- Dedicated customer platform lines justify higher capital allocation at volume

## 2.3 Quanta Cloud Technology (QCT)

Company Overview:

- Market share: ~1 of every 7 servers manufactured globally

- Locations: Taiwan (Taoyuan), US (San Jose CA, Seattle WA), China, Japan, Germany

Key Capabilities:

- Design, manufacturing, integration, and services for servers, storage, and network solutions

- Full-spectrum products and services direct to cloud providers and enterprises

- Rack integration and assembly as core capability

Lessons for GPW:

- OEM-direct model (bypassing channels) provides higher margin

- Co-location near hyperscaler data centers reduces transportation time

- Global scale enables negotiating better component pricing

## 2.4 Foxconn (Hon Hai Technology Group)

Business Model:

- Vertical integration: Board fabrication → module testing → cabinet/rack assembly → final rack integration

- AI server revenue growth: 200%+ (2024–2025)

- Market share target: 40%+

U.S. Expansion:

- Texas: Multiple leased plants for AI server capacity

- Wisconsin: Google server component assembly

- Mexico: Manufacturing and module integration lines

- Total investment: $5 billion committed to U.S. plants

Key Differentiation:

- Vertical control from board fab through final rack assembly eliminates external dependencies

- Shorter lead times vs. multi-vendor orchestration

- Automation targeting 15% unit cost reduction

- OpenAI partnership: Co-design and manufacturing of servers and data center hardware

Lessons for GPW:

- Even partial vertical integration (e.g., cable harness) reduces lead time significantly

- Parallel production lines support multiple customer platforms

- Cost reduction through automation is continuous improvement lever

## 2.5 Celestica (Monterrey, Mexico — DIRECT PRECEDENT)

Monterrey Facility (Established Precedent):

- Location: Calle Octava #102, Parque Industrial Monterrey Apodaca, Nuevo Leon C.P. 66600

- Established: January 1, 1998 (first Mexico presence, acquired from Lucent Technologies)

- Certifications: IATF 16949 (automotive-qualified)

- Services: Printed circuit assembly, systems assembly and box build, value-add engineering, supply chain management, test and repair services, materials laboratory and test development

Data Center Rack Integration Capability:

- Full expertise across all phases of rack integration

- Liquid cooling system design and integration (battery modules, racks, enclosures)

- One of first North American integrated battery module, rack, and enclosure production facilities

- Close proximity to growing customer market segments

Key Differentiation:

- Established 25+ year presence in Mexico with IATF certification

- Integrated manufacturing: Battery modules + racks + enclosures in single facility

- Supply chain flexibility: San Jose NPI center + Monterrey production

- High-reliability industries: Aerospace, defense, and energy sector focus

Lessons for GPW:

- Monterrey location is proven viable for precision assembly (Celestica success)

- IATF 16949 achievable in Mexico; pursue early for automotive/aerospace positioning

- Battery module integration is emerging high-margin opportunity (liquid cooling systems)

- 25+ years of precedent proves skilled workforce availability

## 2.6 Inventec (New U.S. Nearshore Model)

U.S. Facility — Katy, Texas:

- Leased space: ~540,000 sq ft total dry warehouse

- Phase I: 39,390 sq ft converted to assembly facility

- Start: September 15, 2025 | Completion: October 31, 2025

- Focus: AI server assembly

- Initial investment: $85 million total; Phase I = $5 million renovation

Mexico Expansion:

- Ciudad Juárez: Third manufacturing plant planned with $300 million investment

- Strategy: Nearshoring hub for North American demand

Key Differentiation:

- New to U.S. market: First-mover advantage in onshoring AI server production

- Asia-Mexico-USA triangle: Vertically integrated supply chain from Taiwan to U.S./Mexico

- Targeted high-margin: Focus on AI servers rather than commodity servers

Lessons for GPW:

- Nearshoring model (Mexico + U.S.) is accelerating industry-wide

- Rapid facility conversion (one month) requires pre-planning and modular approach

- $300M investment in Ciudad Juárez signals competitive market intensity

## 2.7 Flex Ltd

Scale & Capabilities:

- 100+ sites across 30 countries

- Market focus: Cloud, data center infrastructure, telecommunications

Rack Integration & Manufacturing:

- Full scope: L1–L11 assembly (node assembly through final rack deployment)

- Vertically integrated: Fabrication of sheet metal frames → servers → racks → cabling → cooling systems → busbars → power shelves

- Customization: Industry-standard ORv3 to fully customized enclosures for AI/HPC

Key Differentiation:

- One-stop rack integration without external dependencies

- Liquid cooling expertise for emerging HPC/AI requirements

- Regional footprint: Global scale with local responsiveness

Lessons for GPW:

- L1–L11 process ownership enables faster problem-solving

- Sheet metal fabrication in-house reduces lead time and cost

- Liquid cooling integration becoming table-stakes for high-end AI

## 2.8 ZT Systems (Now Sanmina — Acquired Oct 2025)

Facility Overview:

- New Jersey headquarters: State-of-the-art facility with large power and advanced liquid cooling

- Texas facility: Additional manufacturing capacity

- Netherlands facility: European manufacturing hub

Service Model — Rack-Scale Integration:

- Full rack design: Account for safety, shipping, deployment speed

- Manufacturing: Build and integrate complete rack systems (not just components)

- Deployment support: Systems ready for immediate data center integration

- Entire rack + all interconnections + everything connected to it

Post-Acquisition Status (Oct 27, 2025):

- Sanmina: Now owns manufacturing operations

- AMD: Retains design business; partners with Sanmina for NPI manufacturing

- Manufacturing remains in U.S. as committed

Lessons for GPW:

- Rack-scale integration (complete system) is fastest deployment model

- Modular approaches slower than integrated systems

- Partnership model (design + manufacturing separation) can work if well-managed

## 2.9 Manufacturing Levels (L1–L11) — Industry Hierarchy

| Level | Process | Responsibility | Equipment/Skills |
| --- | --- | --- | --- |
| L1 | Component Procurement | Supplier/ODM | Quality parts sourcing |
| L2–L3 | PCBA Assembly | ODM/EMS | SMT lines, wave solder, inspection |
| L4 | Board Test & Repair | ODM/EMS | Functional test fixtures, burn-in chambers |
| L5 | Node Assembly | ODM/EMS | Chassis assembly, cabling, firmware load |
| L6 | System Integration | ODM/EMS | Multi-node configuration, system test |
| L7 | Rack Assembly | ODM/EMS | Cabinet integration, PDU/networking/cabling |
| L8 | Burn-in & Testing | ODM/EMS | Environmental chambers, power delivery |
| L9 | Quality/Validation | ODM/EMS | Functional testing, performance validation |
| L10 | Packaging & Logistics | ODM/EMS | Protective packaging, crating, shipping |
| L11 | Deployment & Integration | OEM/Cloud Provider | Installation, rack integration at data center |

GPW Focus: L5–L10 (node through packaging); outsource L2–L3 (PCBA) unless pursuing vertical integration

## 2.10 Facility Benchmarking Summary

| Metric | Range | Supermicro (San Jose) | Wistron (Dallas) | Celestica (Monterrey) |
| --- | --- | --- | --- | --- |
| Facility Size | 100K–800K sq ft | 200K+ sq ft (Building 21) | Not disclosed | 50K–100K sq ft |
| Employees | 200–2,000+ | 200+ (Building 21) | 500–800 | 300–500 |
| Racks/Month | 50–600 | 600 | Not disclosed | 50–100 (est.) |
| Power Delivery | 1–5 MW | 3 MW (fuel cell) | 2–5 MW | 1–2 MW |
| Simultaneous Burn-in | 20–60 racks | 60 racks | 20–30 | 10–20 |

# 3. PLANT ZONES — COMPLETE SPECIFICATIONS

## 3.1 Receiving & Incoming Inspection

Purpose: Receive, verify, and inspect all inbound components (chassis, power supplies, networking equipment, cables, connectors, sub-assemblies) before they enter the assembly and kitting workflow. This zone prevents defective materials from reaching production.

Square Footage & Dimensions:

- Recommended size: 1,200–1,800 sq ft (112–167 m²) depending on buffer stock levels

- Staging area per dock: 400–600 sq ft (37–56 m²) per dock bay

- Dock bay dimensions: Minimum 14 ft wide × 40 ft deep (4.3 m × 12.2 m)

- Unloading area: 3,000–4,000 sq ft for 2–3 truck bays

Equipment List:

- Powered dock leveler (±2 inch adjustment)

- Dock shelter (weather protection)

- Pallet jacks (2–3 units, 5,000 lb capacity each)

- Staging shelves (48" W × 24" D, adjustable height)

- Barcode scanner (connected to MES)

- Computer workstation for BOL/ASN verification

- Measuring scale (for weight verification)

- Anti-static flooring or conductive mats (for sensitive components)

ESD Requirements:

- Conductive flooring (<10^6 ohms) in areas handling unshielded components

- Grounding straps available for personnel

- Wrist strap compliance: <10 MΩ resistance

HVAC/Environmental Specs:

- Temperature: 15–27°C (standard warehouse range)

- Humidity: 40–60% RH preferred; 30–70% RH acceptable

- Lighting: 300 lux (adequate for document reading and visual inspection)

- No special cooling required; standard ventilation sufficient

Staffing:

- Dock worker (receiving): 1 per dock bay per shift

- Inspector (QA): 0.5–1 FTE per dock (can roam multiple bays)

- Material handler: 1 per 2 dock bays

Adjacency Requirements:

- Directly adjacent to material storage and kitting zone

- Close to MES workstation for immediate documentation

- Staging area must permit easy flow to component storage

Process Flow:

- BOL presented by carrier (15–30 min per truck)

- Physical unloading of components (30–60 min per truck)

- Component staging by type/order (30–45 min)

- Visual inspection (checking for damage)

- Sample test of received goods (5–10% of shipment)

- Quantity verification and documentation

- Move to component storage or kitting area

Cost Estimate (Phase 1): $30,000–50,000 (basic dock setup)

## 3.2 Material Storage & Kitting

Purpose: Organize, store, and stage components into pre-assembled "kits" that will be delivered to assembly workstations in the exact order needed. This zone minimizes wasted motion and searching during assembly.

Square Footage & Dimensions:

- Recommended size: 2,000–3,500 sq ft (186–325 m²)

- Organized into distinct zones:

  - Component storage: 50% (organized by part number, supplier)

  - Kitting assembly benches: 30%

  - Kit staging/holding area: 20%

Equipment List:

- Shelving (5-tier adjustable, 48"W × 24"D): 15–20 units

- Work benches (kitting stations, 36"H × 60"L × 30"D): 3–4 units

- Bin dividers (for fasteners, cables, connectors)

- Labels and label printer (thermal, MES-integrated)

- Pallet jacks and hand carts

- Rolling carts for kit transport to assembly floor

- Handheld barcode scanner (kit verification)

- ESD mats and grounding equipment

ESD Requirements:

- Conductive flooring recommended (addresses sensitive components)

- Wrist straps available at kitting benches

- ESD-safe bins and containers

HVAC/Environmental Specs:

- Temperature: 15–27°C

- Humidity: 30–60% RH (lower end acceptable for dry components)

- Lighting: 300–500 lux (adequate for part reading and counting)

Staffing:

- Material handler/kit builder: 1–2 FTE (prepares kits)

- Component locator: 0.5 FTE (finds parts, replenishes shelves)

- Verification inspector: 0.5 FTE (audits kit completeness)

Adjacency Requirements:

- Adjacent to receiving (fresh stock intake)

- Adjacent to assembly floor (short transport time to stations)

- Close to product engineering area (for BOM updates)

- View of assembly line (for demand-driven replenishment)

Kit Preparation Process:

- Work order received from MES (specifies kit number, part list, assembly sequence)

- Picker locates all components on shelving (using handheld scanner or paper list)

- Components placed in divided tray in assembly sequence order

- Kit builder verifies quantities (count, compare to BOM)

- Kit labeled with work order number, customer, date prepared

- Kit photographed (for rework traceability if needed)

- Kit staged for delivery to assembly line (typically 30–60 min before use)

Point-of-Use Material Delivery:

- Dedicated material handler (POU provider) delivers fresh kits every 30–60 minutes

- Removes empty containers from previous kit

- Uses U-shaped path to serve multiple assembly stations

- Cost: ~1 person per 20–25 operators

Kanban System Integration:

- Color-coded bin labels indicate stock level thresholds

- When bin reaches reorder point, label goes to purchasing

- Electronic Kanban: MES integration tracks real-time inventory

Cost Estimate (Phase 1): $15,000–25,000 (shelving, benches, carts)

## 3.3 Sub-Assembly (Server Node Pre-Build)

Purpose: Assemble individual server nodes before final rack assembly. Typical node assembly includes CPU installation, memory module insertion, SSD/NVMe mounting, heatsink securing, and power connector installation.

Square Footage & Dimensions:

- Recommended size: 2,500–4,000 sq ft (232–372 m²)

- Organized as linear assembly line with 4–8 workstations

- Each workstation: 40"W × 30"D standing height (38–42" high)

- Aisle width (material flow): 48–60 inches (minimum)

Equipment List:

- Assembly benches: 1 per station (4–8 total)

- Anti-fatigue mats: 1 per station

- Overhead power distribution rail with hanging torque screwdrivers

- Component bins (gravity-fed, placed at waist height)

- Barcode scanner per station (for traceability)

- Work lights (task lighting, 500–750 lux)

- Heatsink assembly tools

- Thermal paste dispenser (controlled quantity)

- Power supply tester (at final station)

- Testing power bank (0–20V, 30A adjustable)

ESD Requirements:

- Full ESD Protected Area (EPA) designation required

- Conductive flooring (<10^6 ohms)

- Wrist straps mandatory (0.5–10 MΩ)

- Workstation mats (conductive, <10^6 ohms)

- Grounded tool cords

- Ionization system if humidity drops below 30% RH

HVAC/Environmental Specs:

- Temperature: 15–27°C (precision work tolerates normal range)

- Humidity: 40–60% RH (ionization backup if <30%)

- Lighting: 500–750 lux (detail work, heatsink alignment)

- Airflow: Non-turbulent (avoid component dust disturbance)

Staffing:

- Operator per station: 1 per workstation (4–8 people)

- Supervisor/inspector: 1 per 4–5 stations

- Material handler (POU): 0.5 FTE (kit replenishment every 30 min)

Adjacency Requirements:

- Adjacent to material kitting area (minimal material transport)

- Visible to burn-in load-out (product flow continuity)

- Near quality lab (failed units sent for diagnostics)

- Separate from high-vibration areas (precision assembly)

Assembly Sequence (Typical 4-Station Line):

Station 1 — CPU & Heatsink Assembly (5–7 minutes):

- Receive prepared node chassis

- Install CPU into socket with torque verification (8–12 Nm per spec)

- Apply thermal paste to heatsink (metered dispenser)

- Mount heatsink and secure retention clip

- Torque fasteners in star pattern to 10 Nm

- Visual inspection for heatsink seating

- Pass to Station 2

Station 2 — Memory & Storage Installation (6–8 minutes):

- Install DIMM modules (0–1 Nm, clips snap automatically)

- Install NVMe SSD (M.2 slot, 0.5–1.0 Nm)

- Install SATA SSD if applicable

- Verify all modules fully seated (visual + slight tug test)

- Pass to Station 3

Station 3 — Power & Cabling (8–10 minutes):

- Route power cables from internal PSU to motherboard (24-pin ATX)

- Install CPU power connector (8-pin 12V)

- Route SATA power to storage devices

- Secure cables with velcro ties (prevent vibration noise)

- Insert internal case fans if modular design

- Pass to Station 4

Station 4 — Testing & QC (10–12 minutes):

- Connect to test power supply

- Power-on test (observe LEDs, listen for fans, check temperature)

- Run BIOS POST (Power-On Self Test)

- Verify all memory detected in BIOS

- Check storage devices enumerated

- Firmware load if required (server image deployed)

- Allow 2–3 minute idle cool-down

- Power-off and prepare for rack assembly

- Pass to staging area

Quality Gates:

- CPU installation: Heatsink seated properly, torque verified

- Memory: All modules fully seated, bending/seating verified

- Thermal paste: Even distribution, no overflow on surrounding components

- Power-on: No error codes, fans spin freely

- Firmware: Correct image version, boot success

Cycle Time & Takt Time:

- Total node assembly time: 30–40 minutes per node (all stations)

- Typical takt time target: 18.3 minutes per rack (for 500 racks/month)

- With 4–8 nodes per rack, assembly line must keep pace

- For high-volume: Parallel lines or shift-based feeding

Cost Estimate (Phase 1): $25,000–40,000 (workbenches, tools, test equipment)

## 3.4 Main Rack Assembly (U-Line)

Purpose: Mechanically assemble server chassis into 42U rack enclosures, mount PDUs, route power and network cables, and prepare for burn-in testing.

Square Footage & Dimensions:

- Recommended size: 3,000–5,000 sq ft (279–465 m²)

- U-shaped layout (see Section 5: Material Flow & Plant Layout):

  - Receiving/unloading: 400 sq ft

  - Assembly stations: 2,000–3,000 sq ft

  - Staging/testing area: 600–1,000 sq ft

- Aisle widths: 60–72 inches (minimum) to accommodate rolling racks

Equipment List:

- Assembly benches: 1 per 2 racks being assembled (for cables/connectors)

- Rack stand (holding empty chassis upright during assembly)

- Power distribution units (PDUs): 1–2 per rack

- Network switch: 1 per rack (typically 24–48 port managed switch)

- Server rails: Universal depth-adjustable (supplied with chassis)

- Mounting hardware: Fasteners, brackets, loctite (blue, medium strength)

- Power tools: Cordless drills, impact drivers (torque-controlled)

- Torque wrenches (5–25 Nm range, digital readout)

- Cable trays and management hardware

- Velcro cable ties (reusable)

- Patch cables: Pre-made CAT6A (18–24 cables per rack)

- Patch panels (if custom routing required)

- Work lights and magnification tools

- Testing PDU or temporary power connection

- Environmental monitoring (thermometer, humidity indicator)

ESD Requirements:

- Flooring: Conductive (for handling sensitive components during cabling)

- Wrist straps: Available but not mandatory (most work on assembled chassis)

- Grounding: Cabling work should use grounded workstation

HVAC/Environmental Specs:

- Temperature: 15–27°C

- Humidity: 40–60% RH

- Lighting: 500 lux (cable routing detail work)

- Airflow: Adequate ventilation (minimal)

Staffing:

- Rack assembly technician: 1 per assembly station (4–6 people)

- Cable specialist: 1 FTE (complex cabling, custom harnesses)

- Inspector/supervisor: 1 per 4–5 stations

- Material handler: 1 FTE (delivering sub-assemblies, staging output)

Adjacency Requirements:

- Adjacent to node sub-assembly staging

- Adjacent to cable assembly (if separate)

- Adjacent to burn-in load-in area

- Close to QA/inspection for defect documentation

Assembly Sequence (Typical 8-Rack Assembly Station):

Step 1 — Rack Preparation (10–15 minutes):

- Position empty 42U rack vertically on stable pallet

- Verify level (±0.5° tolerance using spirit level)

- Install temporary bracing (prevent tipping during assembly)

- Install PDU mounting brackets on rear rail

- Install network switch mounting brackets

Step 2 — Rail Installation (25–35 minutes for 8 pairs):

- Install server rails in pairs at designated slots (typically 2U intervals)

- Verify rail spacing (82–84 mm between left and right rails)

- Torque fasteners to 10 Nm (medium strength loctite applied)

- Verify all rails level and parallel

- Gentle push test (rails should not flex)

Step 3 — Server Mounting (32–40 minutes for 8 servers):

- Insert first server into rail pair (2U slots 2–3)

- Align server ears with rail slots

- Push smoothly until rear reaches rail end

- Install mounting brackets (6–8 Nm torque, medium loctite)

- Verify server sits flush, doesn't rock

- Repeat for each additional server (average 4–5 minutes per server)

Step 4 — PDU Installation (8–10 minutes):

- Mount PDU to rear rail bracket

- Secure mounting fasteners (12–15 Nm for heavy unit)

- Verify PDU is level and doesn't block cable routing

Step 5 — Power Cable Routing (15–20 minutes):

- Route 24-pin ATX power cable from PDU to first server

- Secure with velcro tie every 12 inches (avoid tight binding)

- Route second power cable to same server (N+1 redundancy)

- Repeat for each server

- Verify no cables pinched between components

- Minimum bend radius: 1 inch (prevent damage to insulation)

Step 6 — Network Cable Routing (20–25 minutes):

- Mount network switch to rear rail bracket or separate shelf

- Route network patch cables from switch to each server NIC

- Use separate cable tray for data cables (perpendicular to power cables)

- Label each cable on both ends (source and destination)

- Typical 8-server rack: 16–18 network cables (2 NICs per server + management)

Step 7 — Cable Labeling & Documentation (5–10 minutes):

- Label power cables at both ends (PDU outlet and server)

- Label network cables at both ends

- Color-code if multiple identical cable runs

- Document cable runs in work order (for troubleshooting)

- Generate shipping documentation

Step 8 — Power-On Test (10–15 minutes):

- Connect PDU to temporary test power source

- Verify no immediate breaker trip

- Power-on sequentially (one server at a time if needed)

- Observe all fans spinning, LEDs lighting

- Monitor temperature rise (should reach equilibrium ~40–50°C within 5 minutes)

- Listen for abnormal fan noise or beeping

- Allow 10 minutes at idle

- Power-off gracefully

Quality Gates:

- Rail installation: All fasteners torqued, level, parallel

- Server mounting: Flush seating, no rocking, correct spacing

- Cable routing: No pinched cables, proper securing, labeled

- Power-on: No errors, normal temperatures, expected fan operation

Cycle Time & Capacity:

- Total time per rack: 120–150 minutes (2–2.5 hours)

- With takt time of 18.3 minutes (for 500 racks/month), assembly must pipeline units

- Example: With 4–6 parallel assembly stations, throughput = 12–18 racks/day

- For higher capacity (100+ racks/day), parallel lines required

Cost Estimate (Phase 1): $50,000–80,000 (workbenches, tools, PDU, switch inventory)

## 3.5 Cable Management & Harness Assembly

Purpose: Design and assemble custom power and network harnesses, manage cable routing in racks, test cable continuity, and prepare all interconnections before burn-in testing.

Square Footage & Dimensions:

- Recommended size: 1,500–2,500 sq ft (139–232 m²)

- Divided into:

  - Cable assembly benches: 50%

  - Soldering/crimping station: 30%

  - Cable tray and testing station: 20%

Equipment List:

- Assembly benches: 3–4 units (36"H × 60"L × 30"D)

- Cable crimper (for custom connectors)

- Soldering station with fume extraction (if building custom harnesses)

- Cable tester (continuity, network speed)

- Wrapping dispenser (velcro or other securing material)

- Thermal imaging camera (to detect termination issues)

- Cable cutter/stripper kit

- Pre-made cable inventory (CAT5e/CAT6A patch cables, pre-crimped power cables)

- Cable label maker (thermal printer)

- Cable management trays and clips

- ESD mats and grounding equipment

ESD Requirements:

- Full ESD Protected Area designation (soldering/crimping exposes components)

- Conductive flooring mandatory

- Wrist straps and ESD smocks required

- Soldering irons grounded

- Ionization system if humidity <30% RH

HVAC/Environmental Specs:

- Temperature: 18–25°C (soldering work sensitive to thermal variation)

- Humidity: 40–60% RH

- Lighting: 750–1,000 lux (detail work with small connectors)

- Fume extraction: Mandatory for soldering station (capture flux fumes)

- Ventilation: Dedicated supply/return for this area

Staffing:

- Cable assembly technician: 1–2 FTE

- Crimping specialist (if custom harnesses required): 0.5 FTE

- Soldering specialist (if necessary): 0.5 FTE

- Testing/QA: 0.5 FTE (cable continuity and speed verification)

Adjacency Requirements:

- Adjacent to rack assembly (deliver completed harnesses just-in-time)

- Close to quality lab (failed cables tested for diagnosis)

- Separate from general assembly (soldering fumes shouldn't drift)

- Near burn-in load-in (staging completed harnesses)

Cable Specifications by Type:

Power Cables:

- IEC C13 (10A, 12 AWG): 0.50–1.00 per foot

- IEC C19 (16A, 10 AWG): 0.75–1.50 per foot

- Typical server: 5–20 kW draw (21–83A @ 240V, requires 8–10 AWG)

- Redundant power: N+1 configuration (2 cables per server)

Network Cables:

- CAT6A standard (10 Gbps): $3–5 per pre-made cable

- Fiber LC/LC uplinks (high-speed): $20–40 per cable

- Typical rack: 16–18 network cables (2 NICs × 8 servers + management)

Cable Management Best Practices:

- Separate power and data trays (prevent interference)

- Perpendicular crossing when necessary (minimize electromagnetic coupling)

- Velcro ties instead of plastic (reusable, easier reconfiguration)

- Standard cable lengths minimize slack and waste

- Label cables on both ends with customer/position

- Documentation of all cable runs (for future troubleshooting)

Quality Checks:

- Visual inspection: No damaged insulation, clean connections

- Continuity test: All wires properly terminated

- Speed test (for Ethernet): Verify CAT6A speed if high-speed required

- Thermal imaging: Check terminations for hotspots (poor crimping/soldering)

- Documentation: All cables labeled and documented

Cost Estimate (Phase 1): $20,000–35,000 (tools, testing equipment, pre-made cable inventory)

## 3.6 Firmware Loading & Configuration

Purpose: Load server operating system/firmware, configure BIOS settings, perform initial validation testing, and ensure system readiness before burn-in.

Square Footage & Dimensions:

- Recommended size: 800–1,500 sq ft (74–139 m²)

- Organized as: 1–2 firmware loading stations + testing area

- Equipment staging: Minimal space required

Equipment List:

- Workstations (2–4 units): Standard desktop PC or thin client

- Network switch: 48-port managed (for bulk loading)

- USB drives with OS/firmware images

- Configuration management system (Ansible, Foreman, or similar)

- IPMI (Intelligent Platform Management Interface) testers

- Power distribution for connecting servers

- Documentation terminal (access to configuration records)

ESD Requirements:

- Conductive flooring (server handling)

- Wrist straps available

- Grounding: Standard office environment acceptable (no high-density ESD exposure)

HVAC/Environmental Specs:

- Temperature: 18–25°C

- Humidity: 40–60% RH

- Lighting: 300–500 lux (standard office)

- Airflow: Adequate ventilation (no special requirements)

Staffing:

- Firmware specialist: 1–2 FTE

- Configuration technician: 1 FTE

- Testing validator: 0.5 FTE

Process Flow:

Step 1 — BIOS Configuration (10–15 minutes per rack):

- Connect server to network and power

- Access BIOS via KVM console or IPMI interface

- Configure boot order (primary: network PXE boot)

- Set BIOS to match customer spec (memory profile, thermal settings)

- Enable IPMI if required for remote management

- Save and exit BIOS

Step 2 — Firmware/OS Image Deployment (30–60 minutes per rack):

- Boot server from network (PXE)

- Deploy OS/firmware image via network

- Typical deployment: 20–40 minutes (depends on image size, network speed)

- Monitor progress (automated with configuration management tools)

- Verify successful deployment (boot into OS, no errors)

Step 3 — Configuration Validation (15–20 minutes):

- Verify all DIMM modules detected (CPU + Memory info)

- Confirm all storage devices enumerated

- Check network connectivity (both NICs if present)

- Verify firmware version matches customer requirement

- Document system configuration in MES

Step 4 — Functional Testing (20–30 minutes):

- Run memory diagnostic (mem86+ or equivalent)

- Stress test for 5–10 minutes (CPU/memory load)

- Verify temperature monitoring (sensors report correctly)

- Check system logs for any errors

- Confirm remote IPMI access (if required for burn-in monitoring)

Cost Estimate (Phase 1): $15,000–25,000 (workstations, network switch, IPMI tools)

## 3.7 Burn-in & Stress Testing (MOST DETAILED SECTION)

Purpose: Subject fully assembled server racks to extended operation under controlled environmental conditions (typically 48–72 hours of continuous stress) to detect infant mortality failures before shipment to customer data centers. This is the most critical quality gate and primary capacity differentiator.

Square Footage & Dimensions:

- Recommended size (Phase 1): 2,500–3,500 sq ft (232–325 m²) for 20–30 racks

- Recommended size (Phase 2–3): 5,000–8,000 sq ft for 40–60 racks

- Layout: Hot/cold aisle configuration

  - Cold aisle: Conditioned supply air

  - Hot aisle: Exhaust air containment

  - Vertical separation: ~8–10 feet to accommodate rack height + cooling overhead

Environmental Specifications (ASHRAE TC 9.9 Compliance):

Recommended Operating Envelope:

- Temperature: 18–27°C (64.4–80.6°F) — Standard

- Humidity: 40–60% RH

Allowable Operating Envelope (Extended Range):

- Temperature: 15–32°C (59–90°F) — Emergency operations only

- Humidity: 20–80% RH

High-Density AI Server Classes (New ASHRAE Guidelines):

- H1 Envelope: 18–22°C (64–72°F) — High-performance/AI servers

- H2 Envelope: 18–25°C (64–77°F) — New trend

Monitoring Standards:

- Minimum 6 temperature sensors per rack:

  - Top of rack (hot return air)

  - Middle of rack

  - Bottom of rack (cold inlet air)

  - Repeated for front and back

- Humidity sensors at multiple rack levels

- Continuous monitoring throughout burn-in cycle

- Data logging (MES integration) for all parameters

- Alert thresholds (email/SMS if exceeded)

Equipment List (Critical Infrastructure):

Cooling Systems:

- CRAC/CRAH units: 50–60 ton capacity (Phase 1 for 20–30 racks)

  - Size formula: 200 kW IT load ÷ 3.517 kW/ton = 57 tons required

  - Purchase N+1 redundancy (2 units of 50 tons each)

  - Cost: $8,000–12,000 per unit

- Duct work: Hot/cold aisle distribution

- Air containment: Clear polycarbonate panels (hot aisle or cold aisle)

- Pressure monitoring: Sensors to detect aisle blockage

- Cost estimate: $50,000–80,000 (cooling system + ductwork + containment)

Electrical Infrastructure:

- Main PDU rack (3-phase 208V or 240V): 125–250A capacity

- Branch PDUs (per rack): 24–30 outlet monitored units

- UPS system: 250 kVA, 15 min runtime minimum

  - Size formula: Total load × 1.25 = 280 kW × 1.25 = 350 kVA

  - Cost: $15,000–25,000

- Emergency generator (optional, Phase 2): 250–400 kW

- Main disconnect and circuit breakers (NFPA 70 code compliance)

- Power monitoring: Amp meter on main PDU, per-outlet monitoring

- Cost estimate: $40,000–60,000 (PDUs, UPS, wiring, breakers)

Environmental Monitoring & Control:

- Temperature/humidity sensors: 1 per 100 sq ft minimum

- Data logging system (connected to MES): $2,000–5,000

- Alerting system (email/SMS on threshold breach): Integrated with MES

- 24/7 monitoring display (large screen for operator visibility): $500–1,000

Fire Suppression System:

- Agent type: Novec 1230 recommended (GWP=1 vs. FM-200 GWP=3,500)

- System design: 500–1,000 cu ft coverage (depends on room volume)

- Discharge time: <10 seconds

- Clean agent: No residue, safe for equipment

- Cost: $20,000–40,000 (system + installation)

- Maintenance: Annual inspection, quarterly testing

Burn-in Rack Configuration:

- Typical arrangement: 20–30 racks per burn-in chamber (Phase 1)

- Hot/Cold Aisle Containment:

  - Option 1 — Hot Aisle Containment (HAC): Seal hot aisle from room

  - Exhaust air contained overhead, return through ceiling plenum

  - More complex, more efficient (10–35% energy savings)

  - Suitable for large, centralized burn-in chambers

  - Option 2 — Cold Aisle Containment (CAC): Enclosed cold aisle with sliding doors

  - Simpler to implement, easier to add/remove racks

  - Recommended for modular/growing facilities

  - Recommended for GPW Phase 1–2

Power Density Calculations (Example Scenario):

For 20 racks with loaded servers:

- Conservative: 5 kW per rack (older/entry-level servers)

- Typical: 10 kW per rack (current generation AI servers)

- High-density: 20–30 kW per rack (latest AI/HPC)

Typical Scenario (10 kW per rack):

- 20 racks × 10 kW = 200 kW total IT load

- PDU overhead: 200 kW × 1.05 = 210 kW

- Cooling overhead: 210 kW ÷ 0.5–0.6 PUE = 350–420 kW

  - PUE = Power Usage Effectiveness (industry avg 1.5–2.0; efficient = 1.3–1.5)

UPS Sizing:

- Formula: UPS Capacity = (Total Power Load) × 1.25 (sizing factor)

- Example: 280 kW total × 1.25 = 350 kW minimum

- Recommended: 350–400 kVA (accounting for power factor 0.8–0.9)

- Runtime: Minimum 10–15 minutes (allows generator startup)

- Cost: $15,000–25,000

Cooling Requirement Formula:

- Tons of Cooling = (IT Load in kW) × 3.517 kW per ton ÷ (Cooling efficiency)

- Example: 200 kW IT load × 1.2 = 240 kW cooling load

- CRAC unit sizing: 20–60 tons per unit

- For 240 kW: Need approximately 4–5 CRAC units (60 tons each)

  - OR 2–3 larger units (85+ tons each)

Redundancy Consideration:

- N+1 redundancy: UPS and cooling handle failure of one unit

- For 200 kW: 2 × 125 kW UPS units or 2 × 100-ton CRAC units

- Critical for 24/7 operation and long burn-in cycles

Burn-in Test Protocol & Schedules:

Standard Burn-in (Industry Best Practice):

- Duration: 48–72 hours continuous operation

- Infant mortality window: Most failures occur in first 200+ hours

- Testing load: 80–100% CPU, moderate memory/storage load

- Power consumption: 80–90% of max rated power

- Temperature profile: Constant 18–27°C (per ASHRAE TC 9.9)

Accelerated Burn-in (High-Risk Products):

- Duration: 24–48 hours at elevated temperature

- Use: Detect thermal design issues

- Risk: May not catch all defects found in extended burn-in

Typical Production Schedule:

Day 1 (Monday 0800 hrs — Shift 1):

- Load racks into burn-in chamber (60–90 min for 20 racks)

- Verify all power connections

- Start monitoring systems and temperature ramps

- Begin 48-hour burn-in cycle (starts at 0800)

Day 1–2 (Through Day 2 0800):

- Cycle runs through Shift 2 (1600–2400) and Shift 3 (2400–0800)

- Continuous monitoring: Temperature, humidity, power draw, error logs

- Automated alerts if parameters exceed thresholds

- Shift supervisor checks chamber every 2–4 hours

Day 2–3 (Through Day 3 0800):

- Continue 48-hour cycle through Shift 1 again

- Reach 48-hour mark: ~1600 on Day 2 (or 0800 on Day 3 if 72-hour cycle)

- If 72-hour cycle: Continue to Day 3 0800

Day 3 (Wednesday 0800–1600 — Unload & Test):

- Complete burn-in cycle

- Allow 30-minute cool-down before unloading

- Unload racks (60–90 min for 20 racks)

- Visual inspection during unload (check for loose cables, evidence of overheating)

- Connect to test harness: Run final validation tests (memory, storage, network)

- Pass/Fail determination:

  - PASS: Zero errors in burn-in logs, all final tests successful → Packaging

  - FAIL: Any errors logged or failed tests → Rework/diagnosis

Average Throughput:

- 1 batch per 2.5–3 days (allowing for cycle time + changeover + cool-down)

- For 500 racks/month: Need ~46 rack positions (20–30 racks/batch × 2 batches/cycle × multiple cycles)

- Recommended sizing: 24–30 positions provides flexibility

Burn-in Monitoring Systems:

Real-Time Data Collection:

- Temperature/humidity logged every 5–10 minutes per sensor

- Power measurements: Watts, voltage, current per PDU outlet

- Server health (if IPMI enabled): CPU temp, fan speed, error counts

- All data time-stamped and correlated to specific racks

Alerting & Escalation:

- Alert threshold: Temperature >30°C (activate cooling assistant)

- Alert threshold: Humidity >70% RH (activate dehumidifier)

- Alert threshold: UPS battery critically low (<5 min runtime)

- Alert threshold: Power draw >150% expected (possible short or failure)

- Escalation: SMS/email to on-call technician if thresholds breach

- Automatic logging for post-incident analysis

Capacity Planning & Scaling:

| Phase | Period | Racks/Month | Simultaneous Burn-in | Square Footage | Power (kW) | Cost |
| --- | --- | --- | --- | --- | --- | --- |
| Phase 1 | Year 1 | 50–100 | 10–20 | 2,500–3,500 | 300–400 | $100K–150K |
| Phase 2 | Yr 2–3 | 200–300 | 30–40 | 4,000–6,000 | 600–900 | $200K–300K |
| Phase 3 | Yr 4–5 | 400–600 | 50+ | 6,000–8,000 | 1,000–1,500 | $350K–500K |

Cost Estimate (Phase 1): $150,000–200,000 (cooling, electrical, monitoring, fire suppression, ductwork)

## 3.8 Final Inspection & QC

Purpose: Perform comprehensive visual, functional, and compliance verification of all assemblies before packaging and shipment. This is the final quality gate.

Square Footage & Dimensions:

- Recommended size: 1,200–2,000 sq ft (112–186 m²)

- Divided into:

  - Visual inspection station: 600–800 sq ft

  - Functional testing station: 400–600 sq ft

  - Packaging staging: 200–400 sq ft

Equipment List:

- Inspection benches: 2–3 units

- Magnification lamps (10x magnification)

- Thermal imaging camera (detect hotspots)

- Multimeter (electrical verification)

- Network analyzer (cable/network diagnostics)

- Power analyzer (voltage, current, power factor)

- Cable continuity tester

- Documentation camera (for rework photo records)

- Cleaning supplies (lint-free cloths, isopropyl alcohol)

- Checklists and inspection forms (laminated for durability)

ESD Requirements:

- Conductive flooring recommended

- Wrist straps available (cables may be handled)

- Grounding: Standard office environment acceptable

HVAC/Environmental Specs:

- Temperature: 15–27°C

- Humidity: 40–60% RH

- Lighting: 500–750 lux (detail work)

Staffing:

- QA Inspector: 2–3 FTE (experienced with electrical/mechanical standards)

- Test Technician: 1 FTE (functional validation)

- Documentation: 0.5 FTE (traceability records)

Inspection Checklist (Comprehensive):

Visual Inspection (10–15 minutes per rack):

- [ ] All servers mounted securely (no rocking, fasteners tight)

- [ ] Cable routing clean and organized

- [ ] No visible damage to components or housing

- [ ] All labels present and readable (rack ID, customer info)

- [ ] PDU mounted securely

- [ ] Network switch securely mounted

- [ ] No loose fasteners or hardware

- [ ] Cooling vents clear and unobstructed

- [ ] All cable connections fully seated

Mechanical Verification (5–10 minutes):

- [ ] Rail fasteners torqued (spot-check with wrench)

- [ ] Server mounting brackets secure

- [ ] No visible flex or movement when gentle pressure applied

- [ ] Cable ties secured, no evidence of coming loose

- [ ] Grounding connection verified (multimeter continuity test)

Electrical Verification (10–15 minutes):

- [ ] Power draw at idle: Within expected range (±10%)

- [ ] All server LEDs lit normally

- [ ] Network switch operational (verify with ping)

- [ ] All fans spinning smoothly (listen for grinding)

- [ ] Temperature sensors reporting (if monitored)

- [ ] UPS connectivity verified (if part of customer config)

Functional Testing (15–20 minutes):

- [ ] All servers boot successfully

- [ ] Memory detected correctly in BIOS

- [ ] Storage devices enumerated

- [ ] Network connectivity: All NICs detected and assigned IPs

- [ ] Stress test: Run CPU/memory load 5 minutes, observe temps

- [ ] Firmware version matches customer spec

- [ ] No POST errors or warnings

Network Verification (10–15 minutes):

- [ ] Cable continuity: All 16–18 cables tested for opens/shorts

- [ ] Speed verification: CAT6A cables tested at 10 Gbps (if required)

- [ ] Switch management: Can SSH to switch and verify config

- [ ] Uplink connectivity: Verified to core network (if available)

Documentation & Compliance (5–10 minutes):

- [ ] Work order complete (all steps signed off)

- [ ] Serial numbers of all servers recorded

- [ ] Configuration documented (CPU, memory, storage, firmware version)

- [ ] Burn-in test results reviewed (zero errors required for acceptance)

- [ ] Certifications verified (if ISO, IPC, or industry-specific required)

- [ ] Shipping documentation prepared

Rework Disposition:

- PASS: No issues found → Send to Packaging

- FAIL (Critical): Component failure, shipping hazard, electrical safety issue → Rework required, document in MES

- FAIL (Minor): Cosmetic damage, non-critical function impaired → Customer approval required, proceed or rework per direction

Quality Metrics:

- First-Pass Yield (FPY): Target >95% (racks passing inspection without rework)

- Burn-in Failure Rate: Target <2% (servers failing during 48–72 hour test)

- Final Inspection Failure Rate: Target <1% (issues found in final QC)

Cost Estimate (Phase 1): $15,000–25,000 (inspection benches, test equipment)

## 3.9 Packaging & Crating

Purpose: Protect fully assembled and tested server racks for safe transportation via truck or air freight without damage. Packaging must account for weight distribution, shock protection, and environmental protection during transit.

Square Footage & Dimensions:

- Recommended size: 2,000–3,000 sq ft (186–279 m²)

- Divided into:

  - Material storage (blankets, foam, crating materials): 50%

  - Packaging workstations: 30%

  - Crated goods staging: 20%

Equipment List:

- Work benches: 2–3 units for packing operations

- Cardboard boxes (sized for individual components if needed)

- Foam inserts (pre-formed or custom cut)

- Corrugated crating material

- Wood pallets (standard 48"×40" or customer spec)

- Shrink wrap rolls and heat shrink tools

- Tape dispensers and packing tape

- Labeling equipment (customer labels, shipping labels, QR codes)

- Packing slips and documentation envelopes

- Forklift (in adjacent staging area)

- Weight scale (verify package weight vs. shipping limits)

ESD Requirements:

- Conductive anti-static bags for sensitive items (optional, depends on customer)

- Standard packing acceptable (racks fully assembled/tested, not sensitive)

- Moisture barriers if shipping to tropical climates (Mexico → USA)

HVAC/Environmental Specs:

- Temperature: 15–27°C

- Humidity: 40–60% RH

- Lighting: 300–500 lux

- Ventilation: Adequate (minimal)

Staffing:

- Packing technician: 2–3 FTE

- Material handler: 1 FTE (sourcing packing materials, moving crates)

- Inspector: 0.5 FTE (verify packaging integrity before shipment)

Packaging Sequence (Typical Rack):

Step 1 — Preparation (10 minutes):

- Inspect rack one final time (no pending issues)

- Clean exterior: Remove any dust or labeling residue

- Verify all documentation is inside crate or attached

- Move rack to packaging station (forklift or roller)

Step 2 — Protective Wrapping (15–20 minutes):

- Wrap entire rack with foam edge protection (corners and protruding parts)

- Apply corner protectors (foam or cardboard tubes)

- Wrap entire rack with 2–3 layers of bubble wrap or equivalent

- Ensure cable connectors are protected (wrapped separately)

- Protect door handles and other protruding hardware

Step 3 — Crating (30–45 minutes):

- Build wooden crate around wrapped rack (custom sizing per rack dimensions)

- Typical dimensions: 42"W × 48"D × 78"H (for 42U rack)

- Wood framing: 2×4 lumber, reinforced for forklift mounting

- Interior padding: 2–3" foam on all sides

- Ventilation: Small holes on sides (prevent moisture buildup)

- Secure rack to pallet base with bolts/straps (prevent shifting during transport)

Step 4 — Labeling & Documentation (10 minutes):

- Attach customer shipping label (with PO number, destination address)

- Attach QR code (links to build record and test results)

- Mark "FRAGILE" and "THIS SIDE UP" clearly on multiple sides

- Include contents label (itemized: how many servers, power supply count, etc.)

- Attach weight label (total crated weight)

- Include packing slip inside crate with documentation

Step 5 — Final Verification (5 minutes):

- Verify weight with scale (confirm crate not exceeding shipping limits)

- Visual check: All edges secured, no gaps, labeling complete

- Document crated rack in MES (tracking/shipping)

- Stage for pickup (dock staging area)

Crate Specifications by Shipping Method:

Ground Truck (Continental U.S./Mexico):

- Crating: Standard wooden crate (2×4 frame, plywood sides)

- Pallet base: Standard 48"×40" 4-way pallet

- Protection: Bubble wrap + corner foam minimum

- Weight limit: 2,000 lbs per crate (check carrier limits)

- Cost per crate: $300–500

Air Freight (International/Expedited):

- Crating: Reinforced plywood crate (3/4" plywood, 2×6 frame)

- Pallet base: Specialized air-cargo pallet (lighter composite)

- Protection: Extra padding (3–4" foam all sides)

- Weight limit: 1,500 lbs per crate (airline dependent)

- Cost per crate: $800–1,200

Cost Estimate (Phase 1): $20,000–40,000 (crating materials, labeling, pallets)

## 3.10 Shipping Dock

Purpose: Stage completed/crated racks for carrier pickup, verify shipments, and maintain order flow to customers.

Square Footage & Dimensions:

- Recommended size: 1,500–2,500 sq ft (139–232 m²)

- Divided into:

  - Staging area: 800–1,000 sq ft

  - Dock bay: 400–600 sq ft

  - Office/documentation: 300–400 sq ft

Equipment List:

- Dock leveler (powered, ±2" adjustment)

- Dock shelter (weather protection)

- Overhead doors (minimum 14 ft wide × 12 ft high for standard crates)

- Pallet jacks: 2–3 units

- Forklift access (adjacent to dock)

- Staging shelves (holding area while awaiting pickup)

- Dock management software (integrated with MES)

- Barcode scanner (for shipment verification)

- Printer (shipping labels, BOL documentation)

- Computer workstation (carrier coordination, documentation)

ESD Requirements:

- None (mechanical/structural zone only)

HVAC/Environmental Specs:

- Temperature: Ambient (exposed to outside air during loading)

- Humidity: Ambient

- Lighting: 300 lux (adequate for shipping dock safety)

- Weather protection: Dock shelter prevents rain/snow damage

Staffing:

- Dock supervisor: 1 FTE (coordinate pickups, documentation)

- Dock worker: 1–2 FTE (move crates, load trucks)

- Inventory/documentation: 0.5 FTE (track outbound shipments)

Process Flow:

Step 1 — Staging & Holding (1–3 days typical):

- Crated racks staged in dock holding area

- Organized by customer and destination

- Held awaiting carrier pickup appointment

Step 2 — Pickup Coordination (1 day before scheduled pickup):

- Confirm carrier pickup appointment time

- Verify crate weight and dimensions

- Prepare Bill of Lading (BOL) and shipping documentation

- Notify customer of tracking number

Step 3 — Loading (30–60 minutes per truck):

- Move crated rack to dock bay (forklift)

- Verify crate integrity and labeling

- Load onto carrier truck (per carrier instructions)

- Dock supervisor and carrier representative sign BOL

- Obtain carrier pickup confirmation

Step 4 — Documentation (10 minutes):

- Update MES with shipment status

- Record carrier and BOL number

- Notify customer of shipment details

- Archive documentation for audit trail

Documentation Required:

- Bill of Lading (BOL)

- Packing list (itemized contents)

- Certificate of Conformance (if required)

- Test results/burn-in report (copy for customer)

- Warranty/support documentation

Cost Estimate (Phase 1): $30,000–50,000 (dock equipment, leveler, doors)

## 3.11 Support Zones (Quality Lab, Tool Crib, Rework, IT/MES, Training, Hazmat, Battery Charging)

Quality Lab:

- Square footage: 1,000–1,500 sq ft (93–139 m²)

- Equipment: Thermal imaging camera, oscilloscope, network analyzer, power supply, test fixtures

- Purpose: Troubleshooting failed components, root cause analysis, validation testing

- Staffing: 1–2 FTE

- Cost: $30,000–50,000

Tool Crib:

- Square footage: 600–1,000 sq ft (56–93 m²)

- Equipment: Tool storage cabinets, calibration station, inventory management

- Purpose: Tool management, inventory, maintenance, calibration

- Staffing: 0.5 FTE

- Cost: $10,000–15,000

Rework Area:

- Square footage: 800–1,200 sq ft (74–112 m²)

- Equipment: Soldering stations, desoldering equipment, component test fixtures, magnification

- Purpose: Repair failed components, board rework, connector re-crimping

- ESD requirements: Full EPA (soldering work)

- Staffing: 1–2 FTE

- Cost: $15,000–25,000

IT/MES Office:

- Square footage: 400–600 sq ft (37–56 m²)

- Equipment: Servers, network switches, computers for MES, networking hardware

- Purpose: Manufacturing execution system, production tracking, data management

- Staffing: 1 FTE (IT/systems admin)

- Cost: $20,000–35,000

Training Area:

- Square footage: 600–1,000 sq ft (56–93 m²)

- Equipment: Training benches, sample racks, demonstration models, video/projection

- Purpose: Operator training, process standardization, new employee onboarding

- Staffing: Part-time (during training cycles)

- Cost: $10,000–15,000

Hazmat Storage:

- Square footage: 200–400 sq ft (19–37 m²)

- Equipment: Fire-rated cabinets, ventilation, secondary containment, emergency spill kit

- Purpose: Safe storage of cleaning solvents, thermal paste, flux, adhesives

- Staffing: Monitored as part of general operations

- Compliance: OSHA/EPA requirements for hazardous materials

- Cost: $5,000–10,000

Battery Charging Station:

- Square footage: 200–300 sq ft (19–28 m²)

- Equipment: Chargers for cordless tools, battery inventory, fire-rated charging cabinet

- Purpose: Maintain charged batteries for cordless drills, impact drivers, tools

- Staffing: Maintained as part of tool crib

- Cost: $3,000–5,000

Total Support Zone Footprint: ~4,000–6,500 sq ft (372–604 m²)

Total Support Zone Cost (Phase 1): $93,000–155,000

# 4. ASSEMBLY WORKFLOW — STEP BY STEP

## Complete Process from Receiving to Shipping

Total Manufacturing Cycle Time (per rack): 120–150 minutes of actual work + 48–72 hours burn-in testing

| Step | Zone | Time (min) | FTE | Key Quality Gates | MES Tracking |
| --- | --- | --- | --- | --- | --- |
| 1. Receiving | Receiving Dock | 15–30 | 1 | Visual inspection, BOL match | BOL/ASN scan |
| 2. Unloading | Receiving Dock | 30–60 | 1 | Component damage check | Quantity verified |
| 3. Incoming QC | Inspection Area | 20–30 | 0.5 | Sample testing (5–10%) | Defect logging |
| 4. Kitting | Material Storage | 30–45 | 1 | BOM completeness, sequence | Kit ID assigned |
| 5. Node Sub-Assembly | Assembly Line | 30–40 | 1 | Power-on test, no beeps | S/N logged |
| 6. Rack Assembly | Main Assembly | 120–150 | 1 | Rail torque, cable routing | Rack ID logged |
| 7. Firmware Load | FW Station | 30–60 | 0.5 | Boot success, config verified | Image version recorded |
| 8. Burn-in Test | Burn-in Room | 2,880–4,320 | 0.1 | Zero error logs, no failures | Continuous monitoring |
| 9. Final QC | QA Station | 30–45 | 0.5 | All tests pass, documentation complete | Final sign-off |
| 10. Packaging | Packaging Area | 60–90 | 1 | Crate integrity, labels correct | Tracking number |
| 11. Staging/Shipping | Shipping Dock | 30–60 | 0.5 | BOL match, pickup confirmed | Shipment date recorded |

Total Time: 120–150 min assembly + 48–72 hr burn-in = ~2.5–3.5 days per rack (end-to-end)

# 5. MATERIAL FLOW & PLANT LAYOUT

## U-Shape Layout Rationale

The U-shaped assembly layout is industry best practice for continuous flow manufacturing:

RECEIVING DOCK (North Side)
        ↓
[Receiving] → [Material Storage & Kitting]
    ↓              ↓
[Inspect]    [Node Sub-Assy]
    ↓              ↓
    └─→ [MAIN RACK ASSEMBLY] ←─┘
            ↓
      [Cable Mgmt]
            ↓
      [Firmware Load]
            ↓
      [Burn-in Room] (Climate Controlled)
            ↓
      [Final QC & Inspection]
            ↓
      [Packaging]
            ↓
[SHIPPING DOCK] (South Side)

Benefits of U-Shape:

- Material flow: Components enter one end, finished racks exit other end

- Supervision: Supervisor positioned in center can oversee entire operation

- Flexibility: Easy to add parallel lines or branch operations

- Lead time: Shortest path from receiving to shipping

- WIP reduction: Continuous flow prevents inventory buildup

## Material Flow Pattern

| Stage | Inbound | Outbound | Qty/Day | Lead Time |
| --- | --- | --- | --- | --- |
| Receiving | Raw components | Kits | 1–2 kits | 1–2 hours |
| Sub-Assembly | Kits | Completed nodes | 8–12 nodes | 1–2 hours |
| Main Assembly | Nodes + PDU/Switch | Assembled racks | 6–12 racks | 2–2.5 hours |
| Burn-in | Assembled racks | Tested racks | 6–12 racks | 48–72 hours |
| Final QC | Tested racks | QC-passed racks | 6–12 racks | 1 hour |
| Packaging | QC racks | Crated shipments | 6–12 racks | 2–3 hours |

## Adjacency Matrix (Which zones should be close together)

| Zone A | Zone B | Distance | Reason | Transport Method |
| --- | --- | --- | --- | --- |
| Receiving | Material Storage | <50 ft | Fresh stock intake | Hand cart, forklift |
| Material Storage | Node Sub-Assy | <100 ft | Kit delivery every 30 min | Hand cart (1 person) |
| Node Sub-Assy | Main Assembly | <100 ft | Staged sub-assemblies | Pallet jack |
| Main Assembly | Cable Mgmt | <75 ft | Just-in-time harness delivery | Rolling cart |
| Firmware Load | Burn-in | <100 ft | Direct staging | Forklift |
| Burn-in | Final QC | Adjacent | Unload→test transition | Forklift |
| Final QC | Packaging | <75 ft | QC racks move directly | Forklift/roller |
| Packaging | Shipping Dock | <100 ft | Staged for pickup | Forklift |

## Total Facility Footprint Summary

| Phase | Total Area | Assembly | Burn-in | Support | Offices/Other |
| --- | --- | --- | --- | --- | --- |
| Phase 1 | 15,000–25,000 sq ft | 8,000–12,000 | 2,500–3,500 | 2,000–3,000 | 2,000–6,500 |
| Phase 2 | 25,000–35,000 sq ft | 12,000–16,000 | 4,000–6,000 | 3,500–5,000 | 5,000–8,000 |
| Phase 3 | 35,000–50,000 sq ft | 16,000–22,000 | 6,000–8,000 | 5,000–8,000 | 8,000–12,000 |

## Aisle Widths & Clearances

- Forklift aisles: 72 inches minimum (12 ft turning radius required)

- Material handling paths: 48 inches minimum (pallet jacks, hand carts)

- Personnel paths: 36 inches minimum (pedestrian egress per OSHA)

- Fire egress: 36 inches clear per building code

- Equipment access: 24 inches minimum clearance on all sides (for maintenance)

# 6. BURN-IN ROOM DEEP DIVE

*(Detailed specifications already provided in Section 3.7 — Burn-in & Stress Testing)*

This section consolidates key burn-in specifications for quick reference:

## Hot/Cold Aisle Containment Design

Principle: Separate hot exhaust air (server rear) from cold supply air (server front) to maximize cooling efficiency.

Layout Strategy:

- Racks oriented alternately: Front-to-front (cold aisle), Back-to-back (hot aisle)

- Cold aisle receives conditioned air from CRAC/CRAH

- Hot aisle collects exhaust air for return to cooling unit

- Efficiency gain: 10–35% cooling energy savings possible

Containment Options:

- Hot Aisle Containment (HAC) — More Efficient:

  - Seal hot aisle from rest of room

  - Exhaust air contained overhead

  - Return air through ceiling plenum

  - More complex, more efficient

  - Suitable for large, centralized burn-in chambers

- Cold Aisle Containment (CAC) — Simpler:

  - Enclosed cold aisle with sliding doors

  - Simpler to implement

  - Effective at preventing hot/cold mixing

  - Easier to add/remove racks

  - Recommended for GPW Phase 1–2 (modular/growing facility)

Implementation:

- Use modular containment panels (clear polycarbonate preferred)

- Doors with quick-open for equipment access

- Flexible to accommodate different rack heights

- Plan for 20–25% additional space for containment structure

## ASHRAE TC 9.9 Thermal Guidelines

Recommended Operating Envelope:

- Temperature: 18–27°C (64.4–80.6°F)

- Humidity: 40–60% RH

Allowable Operating Envelope (Extended Range):

- Temperature: 15–32°C (59–90°F)

- Humidity: 20–80% RH

High-Density Classes (New):

- H1 Envelope: 18–22°C (64–72°F) — High-performance/AI servers

- H2 Envelope: 18–25°C (64–77°F) — New trend in industry

Monitoring Standards:

- Minimum 6 temperature sensors per rack (top, middle, bottom × front/back)

- Humidity sensors at multiple rack levels

- Continuous monitoring throughout burn-in cycle

- Data logging to MES with historical trending

## Power Density & Cooling Calculations

For 20–30 Racks (Phase 1):

- IT Load: 200–300 kW (10–15 kW/rack average)

- Cooling required: 240–360 kW (accounting for PUE 1.5–2.0)

- CRAC sizing: 4–5 units @ 60 tons each OR 2–3 units @ 100+ tons each

- UPS sizing: 250–400 kVA with N+1 redundancy

## Fire Suppression (Novec 1230 Recommended)

| Feature | FM-200 | Novec 1230 |
| --- | --- | --- |
| GWP | 3,500 | 1 (preferred) |
| Atmospheric Lifetime | 33–36.5 years | ~5 days |
| Discharge Time | <10 sec | <10 sec |
| Clean-up | None | None |
| Cost | Lower initial | 20–30% higher |
| Status | Being phased out | Preferred for new installations |

# 7. ESD PROGRAM REQUIREMENTS

## ANSI/ESD S20.20 Overview

Standard: ANSI/ESD S20.20-2021 (current version as of July 1, 2023)

Full Name: Protection of Electrical and Electronic Parts, Assemblies and Equipment (Excluding Electrically Initiated Explosive Devices)

Scope: Applies to organizations manufacturing, assembling, testing, handling, or packaging electrical/electronic parts susceptible to ESD damage (≥100V HBM, ≥200V CDM)

Three Fundamental Control Principles:

- All conductors in environment (including personnel) must be bonded/grounded

- Ionization systems must neutralize charges on necessary non-conductive items

- All levels of management must support and maintain ESD program

Two Main Components:

Administrative Requirements:

- ESD Coordinator appointment (1 FTE minimum for facility >200 employees)

- ESD Control Program documentation (policies, procedures)

- Training and awareness (initial + annual refresh)

- Supplier/vendor requirements (certification chain)

- Internal/external audits (quarterly minimum)

Technical Requirements:

- ESD Protected Areas (EPA) design

- Personnel grounding (wrist straps, heel straps)

- Workstation design and materials

- Packaging and shipping

- Equipment testing/verification

- Ionization (when humidity <30% RH)

Product Classes:

| Class | Environment | Reliability | Examples |
| --- | --- | --- | --- |
| Class 1 | Consumer, general industrial | Standard | General electronics |
| Class 2 | Industrial, commercial infrastructure | Enhanced | Telecom, automotive |
| Class 3 | Mission-critical, aerospace, medical | High reliability | Servers, medical devices |

For Server Rack Assembly: Typically Class 2–3 (enhanced to high reliability)

## ESD Flooring Specifications

Conductive Flooring Requirements:

- Resistance: <10^6 ohms to ground (per S20.20)

- Material options:

1. ESD epoxy coating on concrete

2. Conductive tile over subfloor

3. Conductive rubber matting

4. Conductive laminate

Floor Resistance Testing:

- Measured with 4-point probe method

- Must be tested quarterly minimum

- Document all test results

- Report any areas >10^6 ohms for remediation

Installation Considerations:

- Maintain grounding path to facility ground

- Use conductive adhesive/fasteners

- Seal seams properly (moisture barrier)

- Anti-static properties must persist >5 years typically

- Maintenance: Vacuum with ESD-safe equipment, damp mop with conductive solution

Cost Range:

- ESD epoxy coat: $3–8 per sq ft

- Conductive tile: $8–15 per sq ft

- For 1,000 m² (~10,800 sq ft) EPA: $35,000–130,000 depending on material

## Workstation Grounding & Personnel Protection

Wrist Straps:

- Resistance: 1–10 MΩ (per S20.20)

- One per person working in EPA

- Connection: Conductive band around wrist, cord to workstation ground

- Testing: Quarterly minimum, documented

- Cost: $5–20 per strap (high-volume discount)

Heel Straps (Optional but Recommended):

- Used when standing for extended periods

- Contacts flooring at all times

- Reduces dependence on wrist strap alone

- Recommended for high-reliability work (Class 3)

ESD Smocks & Protective Wear:

- Conductive fibers throughout

- Maintains grounding even if sitting

- Required for high-reliability areas

- Cost: $30–80 per smock

Shoe Requirements:

- ESD-safe shoes: Conductive sole (<10^6 ohms)

- Regular cleaning (oils/dirt reduce conductivity)

- Cost: $50–150 per pair

Workstation Grounding:

- Conductive mat on work surface (<10^6 ohms)

- All tools grounded via cord to mat

- Bench-top power distribution units (PDUs) with ground pin

- Regular testing of mat resistance

## ESD Zones Mapping for Server Assembly Plant

FULL ESD PROTECTION REQUIRED (EPA — ESD Protected Area):

- Cable & Wire Harness Assembly:

  - Handles unshielded circuits, exposed traces

  - Controls: Full flooring, wrist straps, grounded workstations

- Box Build / Sub-assembly (Electronics portion):

  - PCBAs exposed during integration

  - Controls: Full EPA with monitoring

- Rework / Repair Station:

  - Soldering and desoldering exposes components

  - Controls: Full EPA plus enhanced testing

- Quality Lab (where testing sensitive components):

  - Direct component manipulation

  - Controls: Dedicated lab EPA

PARTIAL ESD PROTECTION (Recommended):

- Receiving / Inspection:

  - Initial component handling

  - Approach: Conductive floor, periodic grounding

- Kitting / Staging:

  - Assembling component kits

  - Approach: Conductive workbenches, optional wrist straps

- Raw Material Warehouse:

  - Stored unshielded components

  - Approach: Partial floor, bin-level grounding

NON-PROTECTED AREAS (Standard Environment Acceptable):

- Shipping/Receiving dock (mechanical, structural only)

- General warehouses (non-electronic components)

- Offices, Administrative areas

- Break rooms, Cafeteria

- Material handling paths

## ESD Audit & Verification Procedures

Quarterly Audit Schedule:

- Flooring Resistance Testing:

  - 4-point probe method

  - Minimum 5 points per 100 m²

  - Record resistance of each point

  - Action: Remediate any >10^6 ohms

- Wrist Strap Testing:

  - Test each employee strap

  - Resistance measurement

  - Visual inspection for wear

  - Replace if damaged or >10^6 ohms

- Workstation Grounding:

  - Verify all mats properly grounded

  - Continuity test from mat to facility ground

  - Check all tool cords for damage

- Personnel Training Verification:

  - Confirm training documentation on file

  - Spot-check awareness

  - Retraining if gaps identified

- Documentation Review:

  - Audit trail of all testing

  - Supplier qualification records

  - Non-conformance reports

Annual Comprehensive Audit:

- Third-party ESD auditor (recommended)

- Full system assessment

- ANSI/ESD S20.20 compliance verification

- Cost: $3,000–8,000

Cost Estimates (Annual ESD Program):

- Internal auditing: 40–80 labor hours

- External auditor: $3,000–8,000

- Testing equipment: $2,000–5,000

- Remediation (average): $5,000–15,000

# 8. QUALITY STANDARDS & CERTIFICATIONS

## ISO 9001 — Quality Management System

Scope: Overall facility quality system covering all processes from receiving to shipping

Key Requirements:

- Document control procedures

- Change management process

- Non-conformance handling and corrective actions

- Customer feedback and complaint management

- Supplier quality management

- Internal audits (annual minimum)

- Management review (quarterly minimum)

- Continuous improvement metrics

Certification body: Third-party accredited auditor

Cost: $5,000–10,000 initial certification; $2,000–5,000 annual surveillance

Timeline: 6–12 months from implementation to certification

## IPC-A-610 — Acceptability of Electronic Assemblies

Scope: Visual and mechanical inspection standards for electronics assemblies (cable harnesses, PCBs, solder joints)

Key Standards:

- Solder joint quality (wet, shiny, sufficient fillet)

- Cable preparation (proper stripping, crimping, soldering)

- Cleanliness (no flux residue, foreign material)

- Component placement (proper seating, alignment)

- Damage prevention (no burned components, broken traces)

Certification: Requires inspector training and qualification

Cost: $500–1,000 per inspector certification exam

Timeline: 2–4 weeks per inspector

## IPC/WHMA-A-620 — Cable and Harness Assemblies

Scope: Standards for design, manufacture, testing, and inspection of cable and wire harness assemblies

Key Requirements:

- Connector selection and preparation

- Wire stripping, crimping, soldering techniques

- Cable bundling and strain relief

- Testing requirements (continuity, insulation resistance, high-pot testing)

- Documentation and labeling

Certification: Requires technician training

Cost: $300–800 per technician certification

Timeline: 1–2 weeks training + exam

## Industry-Specific Certifications

Automotive (IATF 16949):

- Requires automotive QMS + specific automotive requirements

- Applies if assembling automotive sub-components

- Cost: $15,000–30,000 initial; $8,000–15,000 annual surveillance

- Timeline: 6–12 months

- Relevance for GPW: If pursuing automotive segment (Tier 2 components)

Medical (ISO 13485):

- Medical device quality management system

- Applies if assembling medical equipment/devices

- Cost: $20,000–40,000 initial; $10,000–20,000 annual

- Timeline: 6–12 months

- Relevance for GPW: If pursuing medical device segment

Aerospace (AS9100):

- Aerospace Quality Management System

- Applies if assembling aerospace sub-components

- Cost: $25,000–50,000 initial; $15,000–25,000 annual

- Timeline: 6–12 months

- Relevance for GPW: If pursuing aerospace segment

General (ISO 14001 — Environmental Management):

- Environmental management system

- Supports sustainability credentials

- Cost: $5,000–10,000 initial; $2,000–5,000 annual

- Relevance: Increasingly required by hyperscaler customers

## Certification Roadmap for GPW

Phase 1 (Year 1):

- ISO 9001 (mandatory base certification)

- IPC-A-610 inspector training

- Internal ESD audit compliance

Phase 2 (Years 2–3):

- IATF 16949 (if automotive focus)

- ISO 13485 (if medical focus)

- IPC/WHMA-A-620 technician certifications

Phase 3 (Years 4–5):

- AS9100 (if aerospace focus)

- ISO 14001 (environmental)

- Supplier certification programs (require similar standards from key suppliers)

Total Certification Cost (5-Year Roadmap):

- Initial certifications: $40,000–80,000

- Annual maintenance: $30,000–60,000 per year

- Training/auditing staff: $50,000–100,000 lifetime

- Total: $200,000–400,000 investment

# 9. ENVIRONMENTAL & SAFETY STANDARDS

## ASHRAE TC 9.9 Guidelines (Already Detailed in Section 6)

Recommended operating envelope for data center cooling:

- Temperature: 18–27°C

- Humidity: 40–60% RH

## HVAC Requirements by Zone

| Zone | Temperature | Humidity | Cooling | Ventilation |
| --- | --- | --- | --- | --- |
| Receiving | 15–27°C | 40–60% | None | Standard |
| Component Storage | 15–27°C | 30–60% | None | Standard |
| Assembly | 15–27°C | 40–60% | None | Standard |
| Cable Assembly | 18–25°C | 40–60% | None | Fume extraction |
| Burn-in | 18–27°C | 40–60% | CRAC/CRAH | Significant |
| Quality Lab | 20–25°C | 40–60% | None | Standard |
| Offices | 18–25°C | 40–60% | None | Standard |

Fume Extraction (Soldering Area):

- Capture velocity: 100–150 feet per minute at source

- Local exhaust hoods: Positioned <12 inches from solder joint

- Ducted to outside or carbon filter system

- Cost: $2,000–5,000 per station

## Lighting Specifications

| Task | Lux Required | Fixture Type |
| --- | --- | --- |
| Visual Inspection | 500–750 | LED panel lights |
| Precision Assembly | 500–750 | Task lighting + overhead |
| Cable Soldering | 750–1,000 | Task light + magnification |
| Component Picking | 300–500 | Overhead lights |
| Electrical Testing | 500 | General lighting |
| Receiving Dock | 300 | Overhead fixtures |
| General Areas | 200–300 | Standard fixtures |

LED Advantages:

- 70–80% energy savings vs. incandescent

- Instant on (no warmup)

- Minimal heat generation

- Long lifespan (50,000+ hours)

- Color temperature: 5000K optimal for detail work

- Cost: $100–200 per fixture (installed)

## NFPA Fire Standards

NFPA 70 — National Electrical Code:

- Electrical system design and safety

- Circuit breaker sizing

- Wire gauge selection

- Grounding requirements

- Cost: Design review by licensed electrician ($2,000–5,000)

NFPA 13 — Installation of Fire Sprinkler Systems:

- If automatic sprinkler system installed (separate from Novec 1230)

- Water-based sprinklers acceptable in non-critical areas

- Novec 1230 preferred for burn-in room (no water damage)

NFPA 79 — Electrical Standard for Industrial Machinery:

- Specific to industrial control systems

- Applies to custom burn-in control panels

## Hazmat Handling

Materials Used in Assembly:

- Thermal paste (non-toxic, manage for cleanup)

- Flux (soldering, chemical safety data)

- Isopropyl alcohol (cleaning, ventilation required)

- Loctite/threadlocker (chemical, skin irritant)

- Batteries (if customer includes UPS batteries)

Storage Requirements:

- Fire-rated cabinet for flammables (isopropyl alcohol)

- Secondary containment for hazmat spills

- OSHA hazard communication (SDS sheets posted)

- Emergency spill kit (absorbent material, neutralizer)

- Cost: $5,000–10,000

Hazmat Training:

- Initial training for all facility employees

- Annual refresher

- Specific training for handling materials

- Cost: $500–1,000 per employee (initial)

## OSHA Requirements

General Safety Program:

- Lockout/tagout (LOTO) procedures for equipment

- Machine guarding (power tools, conveyors)

- Personal protective equipment (PPE)

- Incident reporting and investigation

- Regular safety audits (quarterly minimum)

Electrical Safety:

- Arc flash studies (if >600V equipment)

- Proper PPE for electrical work

- Training for electrocution hazards

- Cost: $3,000–5,000 per arc flash study

Ergonomics (Already Detailed in Section 5.3):

- Workstation height: 38–42 inches for standing work

- Anti-fatigue mats: Mandatory for standing stations

- Material positioning: Within arm's reach

- Rotation schedule: Every 1–2 hours (varies muscle groups)

- Lift-assist devices: For items >50 lbs

# 10. LEAN MANUFACTURING INTEGRATION

## 5S Implementation (Sort, Set, Shine, Standardize, Sustain)

Sort (Seiri) — Remove Unnecessary Items:

- Audit assembly floor: Identify tools/materials not used daily

- Remove excess: Extra fasteners, outdated instructions, broken tools

- Maintain only: Parts for current 1–2 day's production

- Target: 20–30% reduction in floor clutter

Set (Seiton) — Organize Remaining Items:

- Assign locations: Every tool, bin, document has a home

- Label with shadows: Pegboards show exact tool position

- Accessibility: Most-used items at waist height

- Color-code: Tools by function, materials by area

- Target: <30 seconds to locate any tool

Shine (Seiso) — Clean & Maintain:

- Daily cleaning: 15 minutes end-of-shift sweep/wipe

- Remove dust from work surfaces, lights, equipment

- Identify maintenance issues (worn cables, sticky hinges)

- Target: Facility clean enough for management walk-through anytime

Standardize (Seiketsu) — Create Standards:

- Document standard procedures (laminated at stations)

- Visual management: Photos of correct setup

- Regular audits: Verify compliance to 5S standards

- Target: All operators follow same procedure

Sustain (Shitsuke) — Maintain Discipline:

- Monthly 5S audits (scoring system)

- Recognize teams with perfect scores

- Address drift (procedures not followed)

- Continuous improvement: Suggest better methods

- Target: 90%+ compliance to 5S standards after 6 months

Cost & Timeline:

- Implementation: $5,000–10,000 (training, supplies, signage)

- Timeline: 3–6 months to establish habits

- ROI: 15–30% productivity improvement, 20–40% reduction in defects

## Kanban for Kitting

Concept: Pull system where each downstream operation triggers upstream to deliver materials

Implementation:

- Kanban cards: Physical cards (or electronic) showing part number, location, reorder point

- Bins: Color-coded (green=stock OK, yellow=reorder, red=out of stock)

- Reorder trigger: When bin reaches yellow level, card goes to material handler

- Delivery: Material handler delivers fresh stock, removes empty bin

- Frequency: Kanban triggers every 30–60 minutes for active parts

Benefits:

- Reduced WIP inventory (20–40% reduction)

- Just-in-time material delivery

- Less floor space for storage

- Fewer stock-outs

Cost: $1,000–2,000 (Kanban boards, cards, training)

## Poka-Yoke (Error Prevention)

Concept: Design processes to prevent human errors (mistake-proofing)

Examples for Server Assembly:

- Jigs for rail installation (ensure correct spacing automatically)

- Fastener trays: Only hold correct quantity for one assembly step

- Cable labels with photos: Match photo to port before crimping

- Torque wrenches: Set to correct value, beep when torque reached (no over/under torquing)

- Component bins: Sized to hold exactly one assembly kit (no extra components)

Benefits:

- Defect prevention (vs. detection)

- Reduced rework

- Operator confidence

- Higher first-pass yield

Cost: $5,000–15,000 (jigs, fixtures, training)

## Gemba Principles (Mezzanine Offices)

Concept: Management offices located on shop floor (not isolated conference rooms) so leaders can observe operations directly

Implementation:

- Small office(s) elevated on mezzanine (10–15 ft above floor)

- Overlooking assembly lines and burn-in area

- Glass walls (transparency for observation)

- Open-door policy for operator questions

- Daily "gemba walks" (management observing floor ~30 minutes)

Benefits:

- Faster problem identification and resolution

- Improved management-operator communication

- Culture of continuous improvement

- Visible leadership engagement

Cost: $20,000–50,000 (mezzanine construction)

## Takt Time Calculation & Line Balancing

*(Already detailed in Section 5.3 — Workstation Ergonomics)*

Example: For 500 racks/month (23 racks/day), with 420 minutes available per shift:

- Takt time = 420 min ÷ 23 racks = 18.3 minutes per rack

- Each assembly station has ~18 minutes to complete its task

- Stations must balance around takt time (no bottlenecks)

- If station takes >18 min, split task or add resource

- If station takes <18 min, combine tasks or add inspection

# 11. CAPACITY PLANNING

## Throughput Calculations (5–600 Racks/Day Scenarios)

Low Volume (Phase 1): 50–100 racks/month

- Daily target: 2–4 racks/day (assuming 22 working days/month)

- Takt time: 110–200 minutes per rack

- Single assembly line: 1 line adequate

- Staffing: 25–45 technicians total

- Facility: 15,000–25,000 sq ft

Medium Volume (Phase 2): 200–300 racks/month

- Daily target: 10–15 racks/day

- Takt time: 27–42 minutes per rack

- Multiple assembly lines: 2 lines optimal

- Staffing: 250–350 technicians

- Facility: 25,000–35,000 sq ft

High Volume (Phase 3): 400–600 racks/month

- Daily target: 20–30 racks/day

- Takt time: 14–21 minutes per rack

- Multiple assembly lines: 4–6 lines or shift-based

- Staffing: 500–700 technicians

- Facility: 35,000–50,000 sq ft

## Staffing Models

Single Shift (8 hours, 1 shift):

- Suitable for Phase 1 (50–100 racks/month)

- Advantages: Simpler management, lower overhead

- Disadvantages: Limited growth capacity

- Burn-in room: Staffed only during loading/unloading

Two Shifts (16 hours, 2 shifts):

- Suitable for Phase 2 (200–300 racks/month)

- Advantages: Doubles capacity without new facility

- Disadvantages: Shift coordination, 24/7 supervision needed

- Burn-in room: 24/7 monitoring (1 person every 2 hours)

24/7 Operation (3 shifts, 24 hours):

- Suitable for Phase 3 (400+ racks/month)

- Advantages: Maximum utilization

- Disadvantages: Complex shift handoff, higher overhead

- Requires: On-call maintenance, security 24/7

- Burn-in room: Continuous staffed monitoring

## Equipment Scaling

| Phase | Node Assembly | Rack Assembly | Burn-in Capacity | Test Equipment | Total Cost |
| --- | --- | --- | --- | --- | --- |
| Phase 1 | 1 line (4 stations) | 1 line (4 stations) | 20–30 racks | Basic | $150K–250K |
| Phase 2 | 2 lines (8 stations) | 2 lines (8 stations) | 40–60 racks | Enhanced | $400K–700K |
| Phase 3 | 4 lines (16 stations) | 4 lines (16 stations) | 60+ racks | Advanced | $1M–1.5M |

## Growth Phases & Expansion Timeline

Phase 1 (Months 1–12) — Establish Operations:

- Deliver: First customer racks by Month 3

- Ramp: 50–100 racks/month by Month 12

- Capacity: 20–30 simultaneous burn-in racks

- Certifications: ISO 9001, ESD compliance

- Key milestone: Achieve first repeat customer

Phase 2 (Months 13–36) — Scale & Multi-Industry:

- Deliver: Support 200–300 racks/month

- Ramp: Two-shift operation

- Capacity: 40–60 simultaneous burn-in racks

- Add certifications: IATF 16949 or ISO 13485 (industry-specific)

- Key milestone: $5M+ annual revenue

Phase 3 (Months 37–60) — Industry Leadership:

- Deliver: 400–600 racks/month (Supermicro parity)

- Ramp: 24/7 operation, 4–6 parallel assembly lines

- Capacity: 50+ simultaneous burn-in racks

- Add: Liquid cooling integration, partial vertical integration

- Key milestone: $20M+ annual revenue, multiple hyperscaler contracts

# 12. MONTERREY-SPECIFIC CONSIDERATIONS

## Climate & Environmental Factors

Seasonal Climate (Monterrey, Mexico):

- Winter (Dec–Feb): 12–22°C (requires heating to maintain 18–27°C target)

- Summer (Jun–Aug): 30–35°C ambient (aggressive cooling required)

- Humidity: Tropical region, 60–80% RH in summer (dehumidification needed)

HVAC Strategy:

- Summer cooling: CRAC units sized 120–150% of standard calculation

- Winter heating: Supplemental heaters for early morning ramp-up

- Humidity control: Dehumidifiers in burn-in and component storage

- Cost impact: 15–25% higher HVAC costs vs. temperate climate

Water Supply:

- Municipal water quality: Adequate for cooling systems (verify hardness/chlorine)

- Water treatment: May be required for CRAC/CRAH systems (avoid scale buildup)

- Wastewater: Proper disposal of cleaning chemicals (isopropyl alcohol, thermal paste residue)

## Labor Market & Workforce

Skilled Workforce Availability:

- Monterrey has established electronics manufacturing base (Celestica precedent proves viability)

- Technician pool: Available from local vocational schools and competing EMS facilities

- Salary expectations: 30–50% lower than U.S. equivalents

- Labor cost arbitrage: GPW competitive advantage vs. U.S.-based ODMs

Training Investment:

- Initial training: ~40 hours per employee (assembly, ESD, quality)

- Ongoing: Annual refresher + certification maintenance

- Cost: $500–1,000 per employee per year

Retention Strategy:

- Competitive wages (vs. local market, not U.S.)

- Career progression: Technician → Supervisor → Engineer

- Benefits: Health insurance, pension (Mexican legal requirements)

- Turnover target: <15% annually (vs. 25–35% industry average)

## Supply Chain Proximity to U.S.

Geographic Advantage:

- 2,000 km from Mexico City

- 12 hours truck drive to Texas (Dallas, Houston hyperscaler hubs)

- 24 hours to U.S. West Coast (if needed)

- Border crossing: Standard (Nuevo Laredo, Reynosa crossings)

Transportation:

- Ground freight: $0.50–1.00 per lb to U.S. (vs. $2–3 air freight)

- Lead time: Next-day delivery to Texas (vs. 3–5 days from Asia)

- Consolidation: Leverage with other Monterrey suppliers (volume discounts)

Component Supply:

- Major suppliers have Mexico operations (AMD, Intel, Broadcom)

- Local vendors: Some power supplies, cables, connectors available locally

- Global supply chain: Components ultimately sourced from Asia (via Monterrey hub)

## Regulatory Environment

Mexican Labor Law (Ley Federal del Trabajo):

- Minimum wage: Current federal minimum ~$248 MXN/day (~$15 USD)

- Workweek: 48 hours maximum (vs. 40 hours in U.S.)

- Benefits: Health insurance, IMSS (pension), severance

- Severance: 3 months salary + 20 days per year of service (if terminated without cause)

- Cost impact: 20–30% payroll overhead

Environmental Regulations (PROFEPA):

- Hazardous waste disposal: Proper classification and licensed disposal

- Air quality: Emissions monitoring (if applicable)

- Water: Wastewater discharge permits

- Cost: Modest for electronics assembly (vs. chemical manufacturing)

Import/Export:

- USMCA (formerly NAFTA): Tariff-free movement of goods U.S.-Mexico

- Duty deferral: Components imported for assembly, re-exported to U.S. typically qualify

- Documentation: Certificates of origin, proper HS codes for customs

- Cost: Minimal (proper paperwork prevents delays)

Quality & Certifications:

- ISO 9001: Recognized globally, same as U.S.

- IATF 16949: Automotive standard recognized in Mexico

- ESD/IPC certifications: No Mexico-specific differences

- As9100 (aerospace): Emerging (first mover advantage for GPW)

## Nearshore Competitive Advantage

vs. U.S. Manufacturing:

- Lower labor cost: 30–50% savings

- Lower facility cost: Real estate cheaper in Monterrey

- Still "local" to hyperscalers: Truck delivery advantage vs. Taiwan/Asia

- Regulatory compliance: Easier than China; USMCA advantage over non-member countries

vs. Asian Manufacturing (Taiwan, Vietnam):

- Lead time: 12 hours truck vs. 3–4 weeks ocean shipping

- Inventory reduction: Just-in-time delivery possible

- Responsiveness: Quick ramps to customer demands

- Quality perception: Mexico less risk than Vietnam; established Celestica precedent

vs. Chinese Manufacturing:

- Geopolitics: Avoid trade tensions, Taiwan dependency

- Intellectual property: Better IP protection in Mexico than China

- Supply chain: Diversification advantage

# 13. APPENDICES

## Appendix A: Complete Equipment List with Costs

Receiving & Dock ($80,000–130,000):

- Powered dock leveler: $8,000–12,000

- Dock shelter: $5,000–10,000

- Pallet jacks (3×): $2,000–3,000

- Staging shelves: $3,000–5,000

- Barcode scanner system: $2,000–3,000

- Workstation + computer: $2,000–3,000

Assembly Area ($150,000–250,000):

- Workbenches (12×): $12,000–18,000

- Torque wrenches (12×): $1,800–2,400

- Hand tools/supplies: $5,000–8,000

- Power tools: $8,000–12,000

- Test equipment: $10,000–15,000

- PDU inventory: $20,000–30,000

- Network switches: $15,000–25,000

- Cable inventory: $10,000–15,000

- Anti-fatigue mats: $3,000–5,000

- Overhead power rail: $15,000–25,000

Burn-in Room ($250,000–350,000):

- CRAC units (2×): $16,000–24,000

- Ductwork/installation: $15,000–25,000

- UPS system: $15,000–25,000

- Fire suppression (Novec 1230): $20,000–40,000

- Monitoring system: $5,000–10,000

- PDU & electrical: $20,000–30,000

- Containment panels: $10,000–15,000

- Temperature/humidity sensors: $5,000–8,000

Support Zones ($100,000–180,000):

- Quality lab equipment: $30,000–50,000

- Tool crib setup: $10,000–15,000

- Rework station: $15,000–25,000

- IT/MES server: $20,000–35,000

- Training area: $10,000–15,000

- Hazmat storage: $5,000–10,000

- Mezzanine office: $20,000–50,000

Total Equipment (Phase 1): $580,000–910,000

## Appendix B: ESD Zone Map Template

FACILITY FLOOR PLAN — ESD ZONES
┌─────────────────────────────────────────────────────────┐
│ RECEIVING DOCK                                  [NORTH] │
│ □ Non-EPA (mechanical zone)                            │
└──────────────┬──────────────────────────────────────────┘
               │
        ┌──────▼──────┐
        │ INSPECTION  │  □ Partial EPA (conductive floor)
        └──────┬──────┘
               │
        ┌──────▼──────────────────┐
        │ MATERIAL STORAGE/KITTING │  □ Partial EPA
        └──────┬──────────────────┘
               │
    ┌──────────┴────────────┐
    │                       │
┌───▼────┐            ┌────▼──────┐
│ NODE   │            │ MAIN RACK  │  □ Full EPA (cable assembly)
│ASSEMBLY│ ────────→  │ ASSEMBLY   │     □ Full EPA (rack assembly)
└────────┘            └────┬───────┘
                           │
                    ┌──────▼──────┐
                    │ CABLE MGMT  │  □ Full EPA
                    └──────┬──────┘
                           │
                    ┌──────▼──────┐
                    │ FIRMWARE    │  □ Partial EPA
                    │ LOAD        │
                    └──────┬──────┘
                           │
                    ┌──────▼──────────┐
                    │ BURN-IN ROOM    │  □ Partial EPA (sensitive)
                    │ 20–30 racks     │
                    └──────┬──────────┘
                           │
                    ┌──────▼──────┐
                    │ FINAL QC    │  □ Partial EPA
                    └──────┬──────┘
                           │
                    ┌──────▼──────┐
                    │ PACKAGING   │  □ Non-EPA
                    └──────┬──────┘
                           │
                    ┌──────▼──────┐
                    │ SHIPPING    │  □ Non-EPA
                    └─────────────┘
LEGEND:
□ Full EPA: Conductive flooring, mandatory wrist straps, grounded workstations
□ Partial EPA: Conductive workbenches, optional wrist straps, standard floor acceptable
□ Non-EPA: Standard environment, no special ESD controls

## Appendix C: Burn-in Test Protocol Template

BURN-IN TEST PROTOCOL
Rack ID: _______________  Date Started: _______________
Technician: _____________  Expected Completion: _______________
PRE-BURN-IN CHECKLIST:
[ ] Rack powered on successfully (no breaker trip)
[ ] All servers boot and reach operating system
[ ] Temperature sensors reporting
[ ] Power monitoring connected
[ ] All network connections verified
BURN-IN CYCLE (48–72 hours):
Duration: _____ hours (target: 48–72)
Load: CPU/Memory load (80–100% target)
Temperature target: 18–27°C
Humidity target: 40–60% RH
MONITORING (Every 4 hours during staffed shifts):
┌──────────┬──────┬────────┬──────────┬────────┐
│Time      │Temp  │Humidity│Power (kW)│Notes   │
├──────────┼──────┼────────┼──────────┼────────┤
│0800 (Day 1)│ ___°C│ __% RH│  ___ kW  │        │
│1200      │ ___°C│ __% RH│  ___ kW  │        │
│1600      │ ___°C│ __% RH│  ___ kW  │        │
│2000      │ ___°C│ __% RH│  ___ kW  │        │
│[Continue every 4 hours through end of cycle]│
└──────────┴──────┴────────┴──────────┴────────┘
AUTOMATED MONITORING (Continuous):
[ ] System logs reviewed for errors
[ ] Temperature excursions logged (if any >30°C)
[ ] Humidity excursions logged (if any >70% RH)
[ ] Power anomalies logged
POST-BURN-IN VALIDATION (at completion):
[ ] Cool-down period: 30 minutes minimum
[ ] Final power-on test: All systems boot successfully
[ ] Memory diagnostic: Run 10 minutes (no errors)
[ ] Storage check: All drives enumerated
[ ] Network connectivity: All NICs responding
[ ] Temperature monitoring: Sensors still reporting
RESULT:
[ ] PASS — All parameters within spec, zero error logs → Proceed to packaging
[ ] FAIL — Errors logged or failed post-test → Move to rework, document issue
[ ] CONDITIONAL — Minor issues, customer approval needed before shipment
Supervisor Sign-off: ________________  Time: _______

## Appendix D: Quality Checklists

FINAL ASSEMBLY INSPECTION CHECKLIST:

RACK ID: _______________  DATE: _______________
MECHANICAL INSPECTION (Visual):
[ ] All servers mounted securely (no rocking)
[ ] Rail fasteners tight (spot-check with wrench)
[ ] No visible damage to components or housing
[ ] Cable routing organized and secure
[ ] All fasteners loctited (blue medium strength)
[ ] Cooling vents clear and unobstructed
[ ] Grounding connection verified (multimeter continuity)
ELECTRICAL VERIFICATION:
[ ] Power draw at idle within ±10% of expected
[ ] All server LEDs lit normally
[ ] Network switch operational (ping test)
[ ] All fans spinning smoothly
[ ] Temperature sensors reporting
[ ] No error codes or warnings in BIOS
NETWORK VERIFICATION:
[ ] All 16–18 patch cables tested (continuity + speed if CAT6A)
[ ] Network switch SSH accessible
[ ] IPMI management access working (if required)
[ ] Uplink connectivity verified
DOCUMENTATION:
[ ] Rack serial numbers recorded
[ ] Configuration documented (CPU, memory, storage, firmware)
[ ] Burn-in test results reviewed (zero errors)
[ ] Work order complete and signed
[ ] All certifications verified (ISO, IPC, industry-specific)
[ ] Shipping documentation prepared
FINAL DISPOSITION:
[ ] PASS — No issues, proceed to packaging
[ ] FAIL — Document defect, move to rework area
[ ] Rework required: Issue _______________
QA Inspector: _______________  Date: _______________

## Appendix E: Sources & References

Industry Standards & Best Practices:

- ANSI/ESD S20.20-2021 — ESD Control Program Standard

- IPC-A-610 — Acceptability of Electronic Assemblies

- IPC/WHMA-A-620 — Cable and Wire Harness Assembly Standard

- ISO 9001 — Quality Management System

- IATF 16949 — Automotive Quality Management

- ISO 13485 — Medical Device Quality Management

- AS9100 — Aerospace Quality Management

- ASHRAE TC 9.9 — Data Center Thermal Guidelines

- NFPA 70 — National Electrical Code

- OSHA Regulations — General Industry Standards

Technical References:

- Supermicro Building 21 — Automated Rack Integration Facility (Data Center Frontier)

- Wistron U.S. Manufacturing Expansion (Yahoo Finance, Fort Worth Report)

- Foxconn OpenAI Partnership (StockTwits/Markets)

- Celestica Monterrey Facility (BNAmericas)

- Inventec Katy, TX Facility (DigiTimes, IndexBox)

- Quanta Cloud Technology Manufacturing (DigiTimes)

- Flex Rack Integration Services (Flex.com)

- ZT Systems Sanmina Acquisition (PR Newswire, AMD Newsroom)

- Burn-in Testing Techniques (Tektronix Application Notes)

- Cable Management Best Practices (VCELINK, PhoenixNAP)

- Server Room Design Standards (PhoenixNAP, Sunbird DCIM)

- Lean Manufacturing Principles (Lean Production, Kanban Zone)

- Material Flow Optimization (EFlex Systems)

Mexican Business Environment:

- USMCA Trade Agreement — Tariff-Free Manufacturing

- Mexican Labor Law (Ley Federal del Trabajo)

- PROFEPA Environmental Regulations

- IMSS Social Security System

# CONCLUSION

This comprehensive best practices guide provides GPW with a detailed operational blueprint for establishing a world-class server rack assembly facility in Monterrey, Mexico. By benchmarking against leading ODMs (Supermicro, Wistron, Foxconn, Celestica), implementing industry-standard quality and ESD controls, and leveraging the nearshore advantage, GPW can position itself as a competitive alternative to U.S.-based and Asian manufacturers.

Key success factors:

- Rigorous burn-in testing capacity (20–60 simultaneous racks) as primary differentiator

- Multi-shift, scalable operations supporting 50–600 racks/month growth phases

- Full ESD compliance and quality certifications (ISO 9001, IPC-A-610, industry-specific)

- Lean manufacturing discipline (takt time, Kanban, poka-yoke)

- Skilled workforce development and retention in Monterrey market

- Monterrey-specific optimization (climate, labor, supply chain)

With disciplined execution of these specifications, GPW can achieve:

- Phase 1: Operational viability and first customer wins (Year 1)

- Phase 2: Multi-industry leadership and $5M+ revenue (Years 2–3)

- Phase 3: Hyperscaler-class facility and $20M+ revenue (Years 4–5)

Document Version: 1.0

Date: March 2026

Classification: Internal Reference — Global Precision Works (GPW)

Last Updated: March 6, 2026

Prepared for: Operations, Facility Planning, Engineering, Quality Teams